Chemical Name: Sodium Glucoheptonate

Shipping Name: Sodium Glucoheptonate, CEE*Quest 1000, 1055

CAS Number:  31138-65-5

Hazardous Classification:  None

ENCEE CHEMICAL SALES       EMERGENCY CONTACTS      DATE:02/14/94

P.O.Box 39                 		  252-633-5868

New Bern, NC  28563        	  252-637-4304

The information provided herein is compiled from internal reports and data from professional publications.  IT IS FURNISHED WITHOUT WARRANTY OF ANY KIND.  EXPRESS OR IMPLIED.  It is intended in evaluating the suitability and proper use of the material in manufacturing and the development and implementation of safety precautions and procedures.

_________________________________________________________________

HAZARDOUS INGREDIENTS

Non Hazardous, this product is a derivative of sugars. It is used as a chelating agent in alkaline solutions.

_________________________________________________________________

PHYSICAL DATA

Appearance: dark brown     	pH: 9.0 to 11

Viscosity: about 25cps         	Odor: Ammonical

Boiling Point: about 215F       	Melting Point: N/A

Vapor Density: same as water    	Freezing Point: N/A

Percent Volatile: 52%     Vapor Pressure (mm Hg): N/A	 Evaporation Rate: unknown          Solubility In Water: infinite

Specific Gravity (Water = 1) : 1.2725to 1.2845

_________________________________________________________________

FIRE AND EXPLOSION HAZARD DATA

Flash Point: NA

Auto Ignition Temperature: NA

Lower Explosion Limit:  NA        Upper Explosion Limit: NA

Extinguishing Media:

_X_ Foam                ___ Alcohol Foam

_X_ Dry Chemical        _X_ Water Fog

_X_ Carbon Dioxide      ___ Other ( )

Special Fire Fighting Procedures:  None known

Unusual Fire and Explosion Hazards:  None known, this product is a derivative of sugars and will not burn unless completely dehydrated at elevated temperatures.

_________________________________________________________________

HEALTH HAZARD DATA

ROUTES OF ENTRY

SKIN CONTACT _X_        EYE CONTACT _X_

INHALATION   _NA        INGESTION _X_

EFFECTS OF OVER EXPOSURE

Inhalation:    NA

Eye Contact:  Burns or irritation

Skin Contact:  Irritation

Ingestion:   Treat the same as sodium hydroxide

_________________________________________________________________

## FIRST AID PROCEDURES

Eyes:  Wash with water, treat as a mild alkali, seek medical attention.

Skin:  Avoid exposure to an open wound, wash with water.

Ingestion:  Treat the same as sodium hydroxide, seek immediate medical attention.

Inhalation:  NA

Listed As Carcinogen By

National Toxicology NA      I.A.R.C.    NA

Program:         NA        Monographs:  NA        OSHA: NA

_________________________________________________________________

REACTIVITY DATA

STABILITY -        _X__  Stable          ____ Unstable

Conditions To Avoid: NA

Materials To Avoid:  Metals that react with alkalis, such as aluminum, magnesium, and other reactive metals.

Hazardous Decomposition Products:   NA

Hazardous Polymerization               Conditions For Polymerization

___ May Occur

_X_ Will Not Occur

Incompatibility ( Materials to Avoid )

Water 	Other

NA      	NA

_________________________________________________________________

SPILL OR LEAK PROCEDURES

Precautions:   This should be treated as a mild alkali.

Clean Up Procedure:  Mop up or flush to drain, rinse affected area with water.

Waste Disposal Methods:  This is biodegradable. In the event of a major spill, neutralization to a specific pH before disposal to the sewer is recommended.

_________________________________________________________________

## SPECIAL PROTECTION INFORMATION

Ventilation Type:     NA

Respiratory Protection:   NA

Protective Gloves:  Yes

Eye Protection:     Goggles

Other Protective Equipment:  None required

_________________________________________________________________

STORAGE AND LABELING

Maximum Storage Temperature:  Keep below 212F

Indoors:   NA

Heated:     NA

Refrigerated: NA

Outdoors:    NA

Storage Notes:   Use standard chemical handling procedures, and avoid excessive heat during storage.

_________________________________________________________________

TOXICITY DATA

Oral:   None known.

Dermal:  None known.

Skin Irritation: None known.

Eye Irritation:  Same as an alkali.

Teratogenicity:  None known.

_________________________________________________________________

## MISCELLANEOUS INFORMATION

This product should be handled with the normal care that should be used when handling chemicals.