Chemical Name: Carbohydrate Chelate (sodium salt of sugar-acids)

Shipping Name: CEE*QUEST 70

CAS Number:  31138-65-5 and 8029-43-4

Hazardous Classification:  None

ENCEE CHEMICAL SALES       EMERGENCY CONTACTS      DATE:10/16/95

P. O. BOX 39                 		252-633-5868

New Bern, NC  28560        	252-637-4304

The information provided herein is compiled from internal reports and data from professional publications.  IT IS FURNISHED WITHOUT WARRANTY OF ANY KIND.  EXPRESS OR IMPLIED.  It is intended in evaluating the suitability and proper use of the material in manufacturing and the development and implementation of safety precautions and procedures.

_______________________________________________________________________

HAZARDOUS INGREDIENTS

Non Hazardous, this product is a derivative of carbohydrates. It is used as a chelating agent in alkaline solutions.

______________________________________________________________________

PHYSICAL DATA

Appearance: dark brown             pH: 	8.0 to 10.5

Viscosity: about 75cps @ 20oC      Odor: slight ammonia

Boiling Point: about 215F          Melting Point: N/A

Vapor Density: same as water       Freezing Point: N/A

Percent Volatile: 45%              Vapor Pressure (mm Hg): N/A Evaporation Rate: unknown          Solubility In Water: infinite

Specific Gravity (Water = 1) : 1.292

______________________________________________________________________

FIRE AND EXPLOSION HAZARD DATA

Flash Point: NA

Auto Ignition Temperature: NA

Lower Explosion Limit:  NA        Upper Explosion Limit: NA

Extinguishing Media:

_X_ Foam                ___ Alcohol Foam

_X_ Dry Chemical        _X_ Water Fog

_X_ Carbon Dioxide      ___ Other ( )

Special Fire Fighting Procedures:  None known

Unusual Fire and Explosion Hazards:  None known, this product is a derivative of carbohydrates and will not burn unless completely dehydrated at elevated temperatures.

______________________________________________________________________

HEALTH HAZARDS DATA

ROUTES OF ENTRY

SKIN CONTACT _X_        EYE CONTACT _X_

INHALATION   _NA        INGESTION   _X_

EFFECTS OF OVER EXPOSURE

Inhalation:    NA

Eye Contact:  Burns or irritation

Skin Contact:  Irritation

Ingestion:   Treat the same as a mild alkali

FIRST AID PROCEDURES

Eyes:  Wash with water, treat as a mild alkali, seek medical attention.

Skin:  Avoid exposure to an open wound, wash with water.

Ingestion:  Treat the same as a mild alkali, seek immediate medical attention.

Inhalation:  NA

Listed As Carcinogen By

National Toxicology NA      I.A.R.C.    NA

Program:         NA        Monographs:  NA        OSHA: NA

______________________________________________________________________

REACTIVITY DATA

STABILITY -        _X__  Stable          ____ Unstable

Conditions To Avoid: NA

Materials To Avoid:  Metals that react with alkalis, such as aluminum, magnesium, and other reactive metals.

Hazardous Decomposition Products:   NA

Hazardous Polymerization               Conditions For Polymerization

___ May Occur

_X_ Will Not Occur

Incompatibility ( Materials to Avoid )

Water           Other

NA             NA

______________________________________________________________________

SPILL OR LEAK PROCEDURES

Precautions:   This should be treated as a mild alkali.

Clean Up Procedure:  Mop up or flush to drain, rinse affected area with water.  In the event of a significant spill, notify local, State, or Federal environmental officials.

Waste Disposal Methods:  This is biodegradable. In the event of a major spill, neutralization to a specific pH before disposal to the sewer is recommended. Notify environmental officials before discharge of any significant amount to the sewer.

___________________________________________________________________ ___

SPECIAL PROTECTION INFORMATION

Ventilation Type:     NA

Respiratory Protection:   NA

Protective Gloves:  Yes

Eye Protection:     Goggles

Other Protective Equipment:  None required

______________________________________________________________________

STORAGE AND LABELING

Maximum Storage Temperature:  Keep below 212F

Indoors:   NA

Heated:     NA

Refrigerated: NA

Outdoors:    NA

Storage Notes:   Use standard chemical handling procedures, and avoid excessive heat during storage.

______________________________________________________________________

Toxicity Data

Oral:   None known.

Dermal:  None known.

Skin Irritation: None known.

Eye Irritation:  Same as a mild alkali.

Teratogenicity:  None known.

______________________________________________________________________

Miscellaneous Information

This product should be handled with the normal care that should be used when handling chemicals.