| SECTION I - PRODUCT IDENTIFICATION Chemical Name: Phosphoric Acid Product Name IG-75 CAS Number 7664-38-2 Manufacturer's Name: Encee Chemical Sales Address: P.O. Box 39 New Bern, NC 28560 Emergency Telephone Numbers: Manufacturer (919) 633-5868 CHEMTREC: (800) 424-9300 (24 hr) SECTION II - HAZARDOUS INGREDIENTS/ IDENTITY INFORMATION Material or Component Percentage Hazard Data Phosphoric Acid 75% Oral-Rats LD50 1530 mg/kg SECTION III- PHYSICAL/CHEMICAL CHARACTERISTICS Boiling Point (oF) 260 Specific Gravity (H2O)=1 1.67 Melting Point N/A Solubility in Water Complete Evaporation Rate (Butyl Acetate=1) N/A Percent Volatiles by Volume N/A Vapor Pressure (75oF) < 10 mm Hg Appearance and Odor Green Liquid Acrid Odor Vapor Density (Air=1) N/A SECTION IV - FIRE AND EXPLOSION HAZARD Flash Point Not Combustible Autoignition Temperature N/A Flammable Air Limits, % by Volume N/A Extinguishing Media N/A Special Fire Fighting Procedures Use Self Contained Breathing Apparatus Use Full Acid Resistant Clothing Unusual Fire and Explosion Hazards Toxic and Irritating Phosphorous Oxides May Form Contact with Metals May Cause Formation of Flammable and Explosive Hydrogen Gas SECTION V - REACTIVITY DATA |  |
| --- | --- |
| Stability | Stable |
| Conditions to Avoid | High Temperature -- May Cause Release of Highly Irritating Phosphorous Oxides |
| Incompatibility | Strong Bases |
| Hazardous Decomposition Products | Phosphoric Acid Mist or Fumes, and Phosphorous Oxides |
| SECTION IV - HEALTH HAZARD INFORMATION |  |
| Toxicity/Health Hazard Data | Oral - See Section II Skin - Unknown Eye - Unknown Air - TLV (ACGIH) 1 /mg/m3 for eight hour weighted average; PEL (OSHA) 1/mg /m3 |
| Routes of Exposure | Ingestion - nausea, vomiting, stomach pain, diarrhea and shock, can be fatal Skin Contact - irritant Eye - Severe irritation, permanent damage or blindness Inhalation - Irritation of nose, throat, and respiratory tract |
| Effects of Acute Overexposure | Large amounts of oral dosage can be serious or fatal see ingestion under routes of exposure. |
| Effects of Chronic Overexposure | See effects of acute exposure. |
| Emergency and First Aid Procedures | Ingestion - If patient is conscious give large amounts of water and induce vomiting. Seek medical attention Skin Contact - Flush with water. Remove Contaminated Clothing. Inhalation - Remove to Fresh Air. If discomfort continues seek medical attention. Eye Contact - Wash with water for at least 15 minutes, including under the eyelid. Seek medical attention. |
| Notes to Physician | Advise of Phosphoric Acid Exposure |

| SECTION VII - PRECAUTIONS FOR SAFE HANDLING/SPILL-LEAK PROCEDURES |  |
| --- | --- |
| Procedure for Leak or Spill | Do not allow anyone to come in contact with spilled material. Contain by diking. Prevent large quantities form entering waterways or from contact with vegetation. |
| Neutralizing Chemicals | Soda Ash or Lime |
| Waste Disposal Method | If uncontaminated, recover and reuse. Consult Local, State, Federal Regulations. |
| Precautions for Storage and Handling | Store in a well ventilated location away from fire hazards. Use a container suitable for phosphoric acid. Do not use metals which react with acids including 304 stainless steels as the acid will cause the formation of hydrogen gas. |
| Other Precautions | Use suitable containers. Do not take internally. Avoid eye and skin contact. Avoid inhaling mist. |

| SECTION VIII - PROTECTION/CONTROL MEASURES |  |
| --- | --- |
| Ventilation | Maintain adequate ventilation. See TLV Section IV. |
| Eyes | Wear splash goggles with a face shield. |
| Skin | Wear impervious acid resistant clothing. |
| Respiratory Equipment | NIOSH/OSHA approved dust or mist respirator when the phosphoric acid mist exposure will exceed the OSHA standard of 1 /mg/m3 |
| Other Equipment | A safety shower and eye wash station shall be available where phosphoric acid is used. |

| SECTION IX - SUPPLIER NOTIFICATION |  |
| --- | --- |
| Shipping Name | Phosphoric Acid |
| Hazard Label | Corrosive |
| Freight Classification | Chemicals, Liquid |
| Transportation Guidelines | Use approved shipping containers. |

| SECTION X - REFERENCES |  |
| --- | --- |
| Date Prepared | June, 1991 |