CEE*QUEST 5% MANGANESE

S5Mn70

CEE*QUEST Manganese ( S5Mn70 ) is a manganese chelate prepared from ENCEE's sugar acid based chelating systems.   The manganese is water soluble.  The product can be used as a source of manganese in agricultural and industrial applications.   This product is biodegradable.  This product is not stable in phosphate solutions.

PRODUCT DATA

| Manganese as Mn | 5.00% 0.53 pound of manganese per gallon |
| --- | --- |
| Color | Dark Red |
| Specific Gravity | 1.273 at 72oF |
| Density | 10.60 lbs/gal |
| pH | 6.5 |
| Crystallization Point | Below 40oF |
| Solubility | Miscible with Water, Fluid Fertilizers, Alkaline nitrogen solutions |
| Compatibility | Solids might form in phosphate containing fertilizer solutions |
| Containers | Tank Cars, Tank Trucks, Drums |

10/93