CEE*QUEST 1000

CEE*QUEST 1000 is a mixture of sodium glucoheptonate isomers and oligosaccharides.   The product is intended for use as a metal ion chelating agent.   The product will chelate metals such as calcium, magnesium, iron, aluminum, manganese, zinc and copper. The product is biodegradable.

PRODUCT DATA

| Calcium Chelation | 250 mg calcium carbonate per gram |
| --- | --- |
| Iron Chelation | 950 mg Fe3+ per gram |
| Color | Red Brown |
| Odor | Ammoniacal |
| Specific Gravity | 1.28 at 72oF |
| Density | 10.60 - 10.70 lbs/gal |
| Solids ( Minimum ) | 47% (ASTM-C494) |
| pH | 9 - 11 |
| Crystallization Point | Below 15oF |
| Solubility | Miscible with Water |
| Containers | Drums, Tank Trucks, Rail Cars |

# Typical Applications and Uses

Alkaline metal cleaning, derusting, descaling, and bottle washing.  Metal sequestering applications in textile or cotton processing.  Aluminum finishing, and boiler water treatment.

07/93